Interface Definition Document (IDD)

END-016: COLLISION AVOIDANCE

List of Contents

1.	Introduction	2

1.1.	Functionalities description	2

1.2.	Use case scenarios	2

2.	Responsibility	2

3.	Applicable and reference documents	2

4.	Definition of terms and abbreviated terms	3

5.	Interface end definition	3

5.1.	Interface end description and data definition	3

5.2.	Units of measure including scale of measure	3

5.3.	Tolerances or required accuracies	3

5.4.	In case of using non-SI quantities or units, conversions and conventions	3

5.5.	Interface plane implementation	3

5.6.	Interface end behaviour	4

5.7.	Interface end data	4

6.	Special Remarks	4

7.	Software	4

7.1.	Operating system	4

7.2.	Operating hardware	4

7.3.	Required software libraries	4

7.4.	Required compiler, build environment	5

7.5.	API definition	5

7.6.	Errors and warnings returned	5

7.7.	Map of inputs/outputs	5

## Introduction

### Functionalities description

The software computes the probability of collision (Pc) between two space objects utilizing analytical approximations based on Chan’s method. The formulation relies on the following standard short-term encounter assumptions:

- Gaussian Covariance: The positional uncertainties of both objects are assumed to follow a normal (Gaussian) distribution.

- Linear Trajectories: Relative trajectories are modelled as rectilinear (straight lines) within the encounter region due to the high relative velocity and short duration of the flyby.

- Spherical Objects: Both spacecraft are modelled as spheres, where the collision cross-section is defined by a hard-body radius (R) equal to the sum of their individual radii (R=r1+r2).

Under these assumptions, the three-dimensional (3D) collision probability problem is projected onto a two-dimensional (2D) plane perpendicular to the relative velocity vector, known as the encounter plane (or B-plane). The collision probability is then evaluated by integrating the 2D Gaussian probability density function (PDF) over a circular domain of radius R, centered at the relative position coordinates:

Pc=112πσxσy1-ρ2x2+y2≤R2exp-121-ρ2x-xm2σx2-2ρx-xmy-ymσxσy+y-ym2σy2dx dy

Where the primary inputs required for the computation are the relative covariance matrix and the relative distance vector at the Time of Closest Approach (TCA).

### Use case scenarios

The analytical and data-adaptive nature of the software makes it suitable for a variety of operational and research scenarios within the domain of Space Situational Awareness (SSA):

- Conjunction Assessment for Un-cooperative Objects: For debris-to-debris or debris-to-satellite encounters where active telemetry is unavailable, the software acts as a primary screening tool. By automatically downloading and processing historical TLE series, it provides an automated estimation of the collision probability to identify high-risk conjunctions.

- Satellite Mission Operations & Manoeuvre Planning: Satellite operators can ingest high-precision ephemerides (e.g., SP3 files or custom orbit determination outputs) into the software during a close approach window. The tool allows operators to quantify the risk reduction of potential collision avoidance manoeuvres (CAM) by updating the relative distance vector and assessing the variation in Pc.

- Historical Conjunction Analysis & Benchmarking: Researchers and analysts can utilize the software to reconstruct past close approach events. By comparing TLE-estimated covariances against high-fidelity sparse data, the module serves as a benchmarking platform to evaluate the limits and uncertainties of TLE-driven risk assessment in different orbital regimes (LEO, MEO, GEO).

## Responsibility

Responsible for END-016:

- Lorenzo Mariani (UNIRM1) Leader

- Fabrizio Piergentili (UNIRM1) Co-Leader

## Applicable and reference documents

- STAR_ProductTree

## Definition of terms and abbreviated terms

| Acronym | Description |
| --- | --- |
| Pc | Probability Collision |
| PDF | Probability Density Function |
| TCA | Time of Closest Approach |
| SSA | Space Situational Awareness |
| TLE | Two-Line Element |
| CAM | Collision Avoidance Manoeuvres |
| LEO | Low Earth Orbit |
| MEO | Medium Earth Orbit |
| GEO | Geostationary Earth Orbit |
| API | Application Programming Interface |
| SP3 | Standard Product 3 |
| OD | Orbit Determination |
| TEME | True Equator Mean Equinox |
| GCRF | Geocentric Celestial Reference Frame |
| RAAN | Right Ascension of Ascending Node |
| SSD | Solid-State Drive |

## Interface end definition

### Interface end description and data definition

The interface serves as the primary data ingestion and boundary layer for the Collision Avoidance Module. It accepts orbital state representations, covariance matrices, and physical geometric properties of two candidate space objects (Primary and Secondary) to compute short-term conjunction risks. Data can be supplied via direct programmatic API calls or structured input files (such as TLE sets or SP3 ephemerides).

### Units of measure including scale of measure

To ensure numerical stability and geometric consistency during the 3D-to-2D transformation, the interface enforces strict unit conventions. All internal calculations are performed using the metric system.

| Parameter | Identifier / Symbol | Standard Unit | Scale / Variable Type |
| --- | --- | --- | --- |
| Relative Position Vector | rrel | Meters (m) | Double-precision Float |
| Relative Velocity Vector | vrel | Meters per second (m/s) | Double-precision Float |
| Covariance Matrix Elements | Σ | Square meters (m2) | 3×3 or 2×2 Matrix |
| Hard Body Radius (HBR) | R | Meters (m) | Floating-point Scalar |
| Collision Probability | Pc | Dimensionless | Scalar range 0.0,1.0 |
| Time of Closest Approach | TCA | Julian Date (JD) / UTC | ISO 8601 String or Epoch Seconds |

### Tolerances or required accuracies

A key operational challenge lies in acquiring high-fidelity relative covariance matrices and relative distances. While these parameters are ideally derived from precise Orbit Determination (OD) processes, such data are rarely available in the public domain.

To circumvent this limitation, the software features a module to estimate both the covariance and the relative geometry by analyzing the historical time-series of Two-Line Element (TLE) sets for the objects of interest.

The accuracy of the computed collision probability (Pc) is highly sensitive to the quality of the input orbital data. While the software fully supports TLE-based estimation, the utilization of precise ephemerides (such as the SP3 format) is strongly preferred over standard TLEs to minimize uncertainty and prevent false positives or negatives in conjunction assessments. Numerical evaluation of Chan's analytical series expansion is constrained to a convergence tolerance threshold of ε ≤10-7.

### In case of using non-SI quantities or units, conversions and conventions

When processing legacy TLE data strings, the software executes an automatic preprocessing pipeline to convert non-SI quantities into the internal SI baseline:

- Mean Motion: Converted from revolutions per day (rev/day) to radians per second (rad/s).

- Angular Metrics: Inclination, RAAN, Argument of Perigee, and Mean Anomaly are converted from degrees to radians.

- Reference Frame Alignment: TLE states natively referenced in the TEME (True Equator Mean Equinox) frame are dynamically transformed into the inertial GCRF (Geocentric Celestial Reference Frame) / J2000 frame before computing relative velocity and constructing the encounter plane.

The interface plane, commonly referred to as the Encounter Plane or B-plane, is implemented by defining a 2D coordinate system perpendicular to the relative velocity vector vrel at the Time of Closest Approach (TCA).

The transformation drops the dimension parallel to the relative motion path, effectively creating a spatial projection where the secondary object's path forms a rectilinear cylinder. The 3×3 relative covariance matrix is squashed into a 2×2 matrix on this plane, allowing Chan's analytical integration to occur over the circular cross-section defined by the hard-body radius R.

### Interface plane implementation

### Interface end behaviour

When triggered, the interface executes the following sequential operations:

- Ingestion & Validation: Parses input datasets (TLE, SP3, or state vectors) and runs formatting validation checks.

- State Propagation: Propagates both objects to the nominal TCA to isolate the minimum relative distance vector.

- Covariance Estimation / Extraction: Extracts the covariance or constructs it dynamically via historical TLE trend analysis.

- Coordinate Transformation: Projects the 3D relative states onto the 2D B-plane.

- Analytical Evaluation: Executes Chan's algorithm to resolve the probability integral.

- Output: Returns the resolved Pc along with geometric metadata and terminates the session cleanly.

### Interface end data

The final data output package generated by the interface is delivered as a structured object containing the following keys:

{

"conjunction_summary": {

"tca_utc": "2026-05-20T12:26:25Z",

"collision_probability": 1.45e-05,

"miss_distance_m": 412.35,

"hard_body_radius_m": 15.0,

"estimation_method_used": "TLE_HISTORICAL_FIT",

"b_plane_parameters": {

"sigma_x": 124.5,

"sigma_y": 342.1,

"correlation_rho": 0.125

}

}

}

## Special Remarks

Interfaces in STAR:

- IF-XXX: COLLISION AVOIDANCE (END-016) – Title Module 2 (END-XXX)

## Software

### Operating system

Primary OS: Microsoft Windows 11 (64-bit, Version 22H2 or higher).

Execution Environment: Architecture-native execution within the Windows environment; fully compatible with Windows Subsystem for Linux (WSL2) for automated batch file routing if required.

### Operating hardware

The software is optimized for standard, mid-range desktop or laptop configurations, matching standard commercial computing profiles:

- Processor (CPU): Intel Core i5 / AMD Ryzen 5 (4 physical cores, 2.5 GHz base clock or equivalent 64-bit architecture).

- System Memory (RAM): 16 GB minimum (required to load the MATLAB R2025a environment alongside large historical TLE arrays).

- Storage Allocation: 1 GB of free space on a Solid-State Drive (SSD) for software scripts, local data caching, and ephemeris storage (excluding the base MATLAB installation footprint).

### Required software libraries

The script framework is developed using native MATLAB programming interfaces to minimize heavy external dependencies:

- Core Environment: MATLAB R2025a Base System.

- Required Toolboxes:

  - Statistics and Machine Learning Toolbox: Utilized for rapid evaluation of multivariate normal probability density functions (PDFs) and coordinate-wise error distributions.

  - Aerospace Toolbox (Optional / Recommended): Employed for high-fidelity reference frame transformations (e.g., TEME to GCRF conversion metrics) if custom analytical filters are omitted.

### Required compiler, build environment

- Development Environment: MATLAB R2025a Integrated Development Environment (IDE).

- Compilation Path: The code executes directly as interpreted MATLAB functions (.m scripts).

- Deployment Options: For deployment on target machines lacking a licensed MATLAB installer, the source code is compatible with the MATLAB Compiler, allowing the generation of a standalone Windows executable (.exe) dependent solely on the free MATLAB Runtime R2025a (64-bit) shared libraries.

### API definition

Syntax:

[calculated_pc, b_plane_meta] = computeCollisionProbability(primary_state, secondary_state, combined_hbr)

Computes short-term collision risk via Chan's method.

Input Parameters:

- primary_state: Struct containing fields 'epoch' (datetime), 'state_vector' (1x6 double, m, m/s), and 'covariance' (3x3 double, m^2). Can alternatively contain a TLE history table.

- secondary_state: Struct structured identically to primary_state for the secondary tracking target.

- combined_hbr: Double scalar representing the sum of both hard-body radii (meters).

Output Parameters:

- calculated_pc: Double scalar within bounds [0.0, 1.0] representing the resolved probability.

- b_plane_meta: Struct output containing projected standard deviations (sigma_x, sigma_y), the cross-correlation coefficient (rho), and localized miss distance parameters.

Exceptions Raised:

- STAR – CollisionAvoidance – MismatchedEpochs: Thrown if relative states cannot be synced to a common TCA.

- STAR – CollisionAvoidance – SingularMatrix: Thrown if the projected 2D covariance matrix is non-invertible.

Non-Functional Effects:

- Transient memory usage increases during historical TLE convergence steps (up to 200 MB allocation peak).

Limitations / Not Allowed Functionality:

- The algorithm assumes short-term rectilinear flybys; low relative velocity encounters (less than 50 m/s) are rejected to protect numerical accuracy boundaries.

### Errors and warnings returned

The interface handles operational edge cases using standard MATLAB error reporting and warning behaviours:

- STAR – CollisionAvoidance – InvalidInputFormat (Error): Raised if data types deviate from expected structures (e.g., passing a 3D position vector instead of a full 6-element state vector). Halts execution.

- STAR – CollisionAvoidance – SingularMatrix (Error): Raised if the relative position and velocity vectors collapse onto a plane where the determinant of the 2D projected covariance equals zero, preventing integration.

- STAR – CollisionAvoidance – DataSparsity (Warning): Triggered if a historical TLE analysis contains fewer than three distinct epochs within the sliding window, alerting the operator to degraded accuracy risks.

### Map of inputs/outputs

| Parameter Name | Data Type | Direction | Access Rules | Description |
| --- | --- | --- | --- | --- |
| primary_state | struct | Input | Read-Only | Houses state vector, covariance, or historical tracking rows for target 1. |
| secondary_state | struct | Input | Read-Only | Houses state vector, covariance, or historical tracking rows for target 2. |
| combined_hbr | double | Input | Read-Only | Scaled sum of structural radii () expressed in meters. |
| calculated_pc | double | Output | Write-Only | Dimensionless calculation outcome containing the computed risk value. |
| b_plane_meta | struct | Output | Write-Only | Contains structural output fields detailing the calculated elements on the 2D encounter plane. |