AGENZIA SPAZIALE ITALIANA

CALL FOR PROPOSALS

PATRIMONIO – Preliminary Design of a High-Resolution Optical Constellation for the Monitoring of Major Italian Cultural Heritage Monuments

Reference: ASI-CFP-2026-PATRIMONIO-01

# 1. Background and objectives

Major Italian cultural heritage monuments are visited by millions of people every year and require continuous monitoring to support their conservation, their structural condition assessment and their protection from the effects of urban development, tourism pressure and environmental change. Frequent, high-resolution optical imagery from a dedicated small-satellite constellation can support the national heritage authorities in this task. The Italian Space Agency (ASI) therefore launches this call for the preliminary design of PATRIMONIO, a small-satellite optical Earth-observation constellation dedicated to frequent high-resolution daylight imaging of Italy's major cultural heritage monuments, while remaining as faint as possible for ground-based telescopes, in line with international recommendations to protect dark and quiet skies from the interference of satellite constellations.

# 2. Constellation and mission concept

The area of interest is defined by a representative set of major Italian cultural heritage monuments distributed across the country, including the Colosseum in Rome, the Cathedral (Duomo) of Milan, the Leaning Tower of Pisa, St Mark's Square in Venice, and the archaeological site of Pompeii, spanning latitude 40 deg N to 46 deg N. The constellation shall have at most 12 satellites, in sun-synchronous orbits between 500 km and 1200 km of altitude, for a mission lifetime of five years. The number of satellites and of orbital planes shall be optimised to minimise the revisit time and to maximise the coverage of these monuments, propagating the orbits for at least 7 days with the Earth oblateness (J2) and the atmospheric drag. Each satellite has a mass not exceeding 100 kg, a drag cross-section of 1 m2 and a drag coefficient of 2.2, is three-axis stabilised and points nominally at nadir. Each satellite shall carry a visible-spectrum push-broom camera with a panchromatic Ground Sample Distance of 1.0 m and a swath of 12 km at nadir, acquire images in daylight only and send them to the ground through UHF and S-band links.

# 3. Reference operational cases

The design shall be checked on two reference cases over 48 hours starting on 21 May 2026 at 13:41 UTC. In the first case a satellite images the Colosseum in Rome (latitude 41.8902 deg N, longitude 12.4922 deg E) in daylight, above 10 deg of elevation and with the Sun above the horizon at the target, preferably higher than 20 deg. In the second case a satellite crosses the sky of the ASI Space Geodesy Centre “G. Colombo” at Matera (latitude 40.6492 deg N, longitude 16.7039 deg E, altitude 400 m), home of the Matera Laser Ranging Observatory, at night: it shall be above 10 deg of elevation and illuminated by the Sun, while the Sun is more than 12 deg below the horizon at the site. For each case the applicant shall identify the satellite and the pass among all those of the constellation and shall report the satellite, the date and the start and end time (UTC) of the pass.

# 4. Operations and energy

Each satellite alternates imaging orbits and idle orbits: in a day of 15 orbits, 8 are imaging orbits and 7 are idle orbits, with downlink through the UHF or S-band transmitters. The electrical power budget shall be closed in sunlight and in eclipse for every operating mode (idle, payload acquisition, UHF downlink and S-band downlink), with the duty cycle of each subsystem and a design margin of 10 percent. The daily energy balance shall not be negative and the battery depth of discharge shall not exceed 30 percent in any orbit.

# 5. Verification requirements

Compliance shall be verified by simulation and analysis only. The applicant shall provide evidence of the optimised constellation architecture and its revisit-time and coverage performance over the identified heritage monuments; the propagated orbit and attitude of the satellites and the two reference passes identified in Section 3, with the satellite, date and start and end time reported for each; the electrical power budget of the operational concept described in Section 4; representative simulated imagery of the daylight reference pass over the Colosseum, showing imaging geometry, footprint and ground illumination consistent with the acquisition time; and the light reflected by the satellite during the night reference pass over Matera, expressed as a light curve together with the phase angle, the range and the estimated apparent visual magnitude over the pass. The peak apparent visual magnitude of the satellite seen from Matera shall be fainter than magnitude 7, the conventional limit of naked-eye visibility, and the light-pollution impact of the constellation shall be reported against this limit.

# 6. Deliverables and evaluation

Proposals shall provide the optimised constellation architecture with its revisit time and coverage over the identified heritage monuments; the satellites, dates and start and end times of the two reference passes; the electrical power budget tables and figures; the simulated images of the daylight pass over the Colosseum; and the light curve, apparent magnitude and light-pollution assessment from Matera. Proposals will be evaluated on compliance with the requirements above, on the technical quality of the design and on the lowest impact on astronomy. Costs and lifecycle economics are outside the scope of this call.

This is a simulated call prepared as an example for a mission-analysis exercise; it is not an official ASI document.