AGENZIA SPAZIALE ITALIANA

CALL FOR PROPOSALS

PATRIMONIO-CUBE

Preliminary Design of a CubeSat Optical Constellation for the Monitoring of Major Italian Cultural Heritage Monuments

Reference: ASI-PCUBE-CFP-001

Issue / Revision: Issue 1, Revision 2

Type of activity: Preliminary design study, by analysis and simulation only

Indicative duration: 6 months from kick-off (T0)

Language: English

# 1. Introduction and objectives

## 1.1 Background

Italy hosts one of the largest concentrations of cultural heritage properties in the world. Major monuments require continuous monitoring to support their conservation, the assessment of their structural condition and their protection from urban development, tourism pressure and environmental change. Frequent optical imagery from a constellation of CubeSats built from commercially available components can support the national heritage authorities at a fraction of the cost of larger satellites.

## 1.2 Objectives

The Italian Space Agency (ASI) launches this call for the preliminary design of PATRIMONIO-CUBE, a constellation of CubeSats dedicated to frequent daylight imaging of Italy's major cultural heritage monuments. The constellation shall also remain as faint as possible for ground-based telescopes, in line with international recommendations to protect dark and quiet skies.

## 1.3 Scope of the activity

The activity covers the mission analysis, the optimisation of the constellation architecture, the selection of commercially available platform components and the demonstration, by simulation and analysis, that the design meets the technical conditions described in Sections 4 and 5 on the two reference operational cases of Section 6. The manufacturing of hardware, the procurement of launch services and the development of the ground segment are outside the scope of this call. Costs and lifecycle economics are outside the scope of this call and no financial proposal is requested.

# 2. Applicable and reference documents

The documents listed as applicable (AD) form part of this call to the extent specified in the text. The documents listed as reference (RD) are provided for information. In case of conflict, the text of this call prevails over the applicable documents, and the applicable documents prevail over the reference documents. The latest issue of each document at the date of publication of the call applies.

## 2.1 Applicable documents

AD-01	ECSS-E-ST-10C, Space engineering: System engineering general requirements.

AD-02	ECSS-E-ST-10-04C, Space engineering: Space environment.

AD-03	ECSS-E-ST-10-09C, Space engineering: Reference coordinate systems.

AD-04	ECSS-E-ST-20C, Space engineering: Electrical and electronic.

AD-05	ECSS-E-ST-31C, Space engineering: Thermal control general requirements.

AD-06	ECSS-E-ST-60-10C, Space engineering: Control performance.

AD-07	ECSS-U-AS-10C, Adoption Notice of ISO 24113: Space systems, space debris mitigation requirements.

AD-08	ESSB-ST-U-007, ESA Space Debris Mitigation Requirements.

AD-09	CubeSat Design Specification (CDS), Rev. 14.1, California Polytechnic State University.

AD-10	ITU Radio Regulations.

## 2.2 Reference documents

RD-01	Report of the SATCON1 Workshop on Satellite Constellation Impacts on Astronomy, 2020.

RD-02	Report of the SATCON2 Workshop, 2021.

RD-03	IADC Space Debris Mitigation Guidelines.

RD-04	UN COPUOS Guidelines for the Long-term Sustainability of Outer Space Activities, 2019.

# 3. Terms, definitions and conventions

Requirement verbs. The verb “shall” identifies a mandatory provision. The verb “should” identifies a desirable goal that the applicant is expected to pursue and to justify if it is not met. The verb “may” identifies a permission.

Revisit time. The time interval between two consecutive imaging opportunities of the same target. Mean revisit time is the average of these intervals over the evaluation period, computed over the whole constellation.

Imaging opportunity. A time interval during which at least one satellite can acquire an image of a target while meeting all the geometric and illumination conditions of this call. Cloud cover is not considered.

Coverage. The fraction of days of the evaluation period with at least one imaging opportunity for a given target.

Night. A condition in which the Sun is more than 12 deg below the local horizon at the observer (end of nautical twilight).

Apparent visual magnitude. The magnitude in the visual (V) band of the sunlight reflected by a satellite, as seen by a ground observer at the stated range and phase angle, without atmospheric extinction. A light curve is the apparent visual magnitude as a function of time.

Time and frames. All epochs are expressed in UTC. Coordinates of ground locations refer to WGS84. Orbit and attitude analyses shall use reference frames consistent with AD-03.

Reference epoch (T_ref). 21 May 2026 at 13:41 UTC.

# 4. Mission description

## 4.1 Mission concept

PATRIMONIO-CUBE is a constellation of CubeSats in low Earth orbit dedicated to the frequent imaging of the monuments of Section 4.2. The constellation is expected to serve the national heritage authorities for a mission lifetime of three years, counted from the end of the commissioning phase. Each satellite carries a visible-spectrum push-broom camera, acquires images in daylight only and sends them to the ground through radio links. The type of orbit, the number of satellites and the number of orbital planes are left to the applicant: the architecture shall be selected through a documented optimisation in which the revisit time is minimised and the coverage of the monuments is maximised. The optimisation method, the design variables, the figures of merit and the explored trade space are to be reported.

## 4.2 Reference targets

The area of interest is defined by a representative set of major Italian cultural heritage monuments, spanning latitude 40 deg N to 46 deg N. The reference targets and their coordinates are:

- Colosseum, Rome: 41.8902 deg N, 12.4922 deg E.

- Cathedral (Duomo), Milan: 45.4642 deg N, 9.1916 deg E.

- Leaning Tower, Pisa: 43.7230 deg N, 10.3966 deg E.

- St Mark's Square, Venice: 45.4340 deg N, 12.3388 deg E.

- Archaeological site of Pompeii: 40.7506 deg N, 14.4869 deg E.

## 4.3 Constellation, orbit and space debris

The satellites shall orbit between 500 km and 650 km of altitude, and the applicant shall declare the operational altitude and the orbital elements of every satellite at T_ref. The performance of the constellation is judged on the revisit time and on the coverage of each reference target, evaluated over a period of at least one year in order to account for the seasonal variation of illumination. The mean revisit time of each target shall not be greater than 48 hours, with a goal of 24 hours, and the maximum revisit gap shall also be reported. Each target shall be covered on at least 50 percent of the days of the evaluation period, with a goal of 80 percent.

The orbit propagation shall include at least the Earth's gravity field with the J2 perturbation, the atmospheric drag and the solar radiation pressure, and the models and parameters shall be declared. The orbital lifetime shall be assessed with an atmospheric drag model and with solar and geomagnetic activity predictions consistent with AD-02, and the decay of the orbit over the mission lifetime shall be shown to be compatible with the imaging performance of Section 5.1. The applicant shall describe the deployment and phasing strategy, including the assumed launcher, the dispenser and the injection orbit dispersions.

The satellites shall keep a safe separation from each other: the distance between any two satellites of the constellation shall not fall below 5 km at any time over the mission lifetime, taking into account deployment dispersions and differential drag. The applicant shall also describe the approach to conjunction assessment and collision avoidance, in line with AD-07 and AD-08.

Each satellite shall re-enter the atmosphere within 5 years of the end of its mission, in compliance with the space debris mitigation rules of AD-07 and AD-08, with a probability of successful disposal of at least 0.9. The disposal strategy (passive decay or propulsive manoeuvre) and the corresponding disposal delta-v shall be provided. The casualty risk on ground from any fragment surviving the re-entry shall not exceed 1E-4, and a demise analysis shall support the figure.

## 4.4 Operations concept

Each satellite alternates imaging orbits and idle orbits. In a day of 15 orbits, 8 are imaging orbits and 7 are idle orbits. Payload data are sent to the ground through the UHF or S-band transmitters. As reference ground station for telemetry, telecommand and data downlink the applicant shall assume the ASI Space Geodesy Centre “G. Colombo” at Matera (latitude 40.6492 deg N, longitude 16.7039 deg E, altitude 400 m); additional stations may be proposed with justification.

# 5. Technical description of the system

This section describes the expected technical characteristics of the satellites. The applicant shall treat the statements containing “shall” as binding for the design and shall report, in the compliance statement of Section 7, how each of them is met.

## 5.1 Payload and imaging performance

Each satellite is equipped with a visible-spectrum push-broom camera with a panchromatic Ground Sample Distance of 6.5 m, or better, and a swath of 24 km, or wider, at nadir. At the operational altitude declared by the applicant this implies a detector line of at least 3,700 pixels across track. The panchromatic band shall be contained within 450 nm to 750 nm. The signal-to-noise ratio shall be at least 50 for a surface albedo of 0.3 and a Sun elevation of 30 deg at the target, the modulation transfer function at the Nyquist frequency shall be at least 0.10 and the radiometric quantisation shall be at least 10 bit. These figures shall be justified with vendor datasheets.

The satellite shall be able to image targets at an off-nadir angle of up to 30 deg, with a GSD not exceeding 9 m at that angle. Images are acquired in daylight only: the Sun shall be above the local horizon at the target, and the Sun elevation should be higher than 20 deg. The absolute geolocation accuracy of the images shall be better than 500 m (CE90) without the use of ground control points, and each image shall be available on the ground within 48 hours from its acquisition.

## 5.2 CubeSat platform

Each satellite is a CubeSat in a format between 6U and 12U, compliant with AD-09. The total mass, including a margin of 10 percent, shall remain within the maximum allowed by AD-09 for the selected format, namely 12 kg for 6U and 24 kg for 12U. The platform components are the structure, the on-board computer, the electrical power system, the battery, the deployable solar panels, the UHF and S-band radios, the attitude determination and control subsystem and the optical payload. They shall be selected among commercially available CubeSat products documented by vendor datasheets and shall fit, together, within the mass and volume of the selected format, with a volume margin of at least 10 percent and with the deployable solar panels, stowed, within the dispenser envelope.

For each component the applicant shall provide a bill of materials entry stating vendor, model, mass, dimensions, declared power in each operating mode and operational temperature limits. Each component shall have a Technology Readiness Level of at least 7 or in-orbit heritage, to be declared by the applicant, and its compatibility with the total ionising dose accumulated over the mission lifetime shall be shown, stating the shielding assumptions.

## 5.3 Attitude determination and control

During imaging the pointing accuracy shall be better than 0.1 deg (3 sigma) and the attitude knowledge better than 0.03 deg (3 sigma). The pointing stability during image acquisition shall be better than 0.01 deg/s (3 sigma). The satellite shall be able to slew by 30 deg in not more than 120 seconds, so as to point at targets off the ground track.

## 5.4 Electrical power

The electrical power budget shall be closed in sunlight and in eclipse for every operating mode, namely idle, payload acquisition, UHF downlink and S-band downlink, with the duty cycle of each subsystem in each mode. Power consumption values are taken from the vendor datasheets of the selected components, and any value not available in the datasheets shall be justified. A design margin of 10 percent shall be applied to the budget.

The daily energy balance shall not be negative over a day of 15 orbits with 8 imaging orbits and 7 idle orbits, as described in Section 4.4, and the battery depth of discharge shall not exceed 30 percent in any orbit. Both conditions shall be met in the worst case, at end of life, considering the degradation of the solar cells and of the battery capacity, the maximum eclipse duration and the worst Sun-array geometry of the selected orbit. The power generated by the solar panels shall account for the cell temperature derived from the thermal analysis of Section 5.5.

## 5.5 Thermal behaviour

The temperatures of the CubeSat faces, of the solar panels, of the central body and of the battery shall be assessed along the orbit and during the daylight reference imaging pass of Case A (Section 6.2). The battery temperature shall stay within the operational limit of the selected battery at all times, with a margin of at least 5 deg C at both the lower and the upper limit; where the datasheet gives different limits for charge and discharge, both shall be respected.

The analysis shall consider at least a hot case and a cold case, defined by the solar flux, the Earth albedo and the Earth infrared emission of AD-02 and by the beta-angle extremes of the selected orbit. The thermal model shall be at least a lumped-parameter model with a node for each face, for the solar panels, for the central body and for the battery, and the heat dissipated in each operating mode shall be consistent with the power budget of Section 5.4.

## 5.6 Communications and data handling

Data are sent to the ground through radio links in the UHF band or in the S band, and the applicant shall indicate which link is used for each purpose. The link budget of each link shall be closed with a margin of at least 3 dB at a ground station elevation of 10 deg. The applicant shall provide the contact time with the reference ground station of Section 4.4 and the resulting downlink capacity per day. The data volume generated per day shall not exceed that capacity, with a margin of at least 20 percent, and the on-board storage shall hold at least 48 hours of acquisitions. The selected frequencies shall comply with AD-10, and the applicant shall identify the frequency allocation and the licensing path.

## 5.7 Impact on astronomy

The peak apparent visual magnitude of every sunlit satellite seen from Matera at night shall be fainter than magnitude 7, the conventional limit of naked-eye visibility. The light reflected by the satellite during the night pass of Case B (Section 6.3) shall be provided as a light curve with a time step not larger than 5 seconds, together with the phase angle, the range and the estimated apparent visual magnitude.

The light curve shall be computed with a documented reflectance model for the surfaces of the satellite (solar panels, faces and optical aperture), including diffuse and specular components, and with the attitude of the satellite during the pass. The estimated magnitude shall be accompanied by its uncertainty and by the sensitivity to the main assumptions of the model. The applicant shall describe the design measures adopted to reduce the brightness, such as surface materials and coatings, attitude strategy or operational constraints, together with their impact on the power and thermal budgets. The applicant should also report the number of sunlit passes above 10 deg of elevation seen from Matera at night during the 48 hours of the reference cases, and the peak magnitude of each.

# 6. Reference operational cases

## 6.1 Common conditions

The design shall be checked on two reference cases over 48 hours starting at T_ref. The applicant shall use the constellation defined in the proposal, with the initial conditions declared as per Section 4.3, and shall select the satellite and the pass that satisfy the conditions of each case. If no pass satisfying the conditions of a case exists within the 48 hours, the applicant shall state it and shall report the first pass that does, after the end of the window.

## 6.2 Case A: daylight imaging of the Colosseum

A satellite images the Colosseum in Rome (latitude 41.8902 deg N, longitude 12.4922 deg E) in daylight. The satellite shall be above 10 deg of elevation as seen from the target, the Sun shall be above the horizon at the target, preferably higher than 20 deg, and the off-nadir angle shall respect the limit of Section 5.1. For this case the applicant shall report:

- the satellite, the date, and the start and end times of the pass;

- the imaging geometry (off-nadir angle, satellite elevation and azimuth) and the ground footprint;

- the Sun elevation and azimuth at the target;

- the temperatures of faces, solar panels, central body and battery during the pass.

## 6.3 Case B: night pass over Matera

A satellite crosses the sky of the ASI Space Geodesy Centre “G. Colombo” at Matera (latitude 40.6492 deg N, longitude 16.7039 deg E, altitude 400 m) at night, above 10 deg of elevation and sunlit. For this case the applicant shall report:

- the satellite, the date, and the start and end times of the pass;

- the light curve, the phase angle and the range along the pass;

- the estimated apparent visual magnitude and its peak value.

# 7. Verification and deliverables

## 7.1 Verification

Compliance shall be verified by simulation and analysis only. The applicant shall provide evidence of the optimised constellation architecture and of its revisit-time and coverage performance over the reference targets; of the propagated orbit and attitude of the satellites and of the two reference cases; of the electrical power budget; of the selected platform components with their bill of materials, mass, volume and declared power; of the temperatures of the CubeSat against the battery limit; of representative simulated imagery of the daylight pass over the Colosseum; and of the light curve, the phase angle, the range and the estimated apparent visual magnitude of the night pass over Matera.

The simulated imagery shall show the imaging geometry, the footprint and a ground illumination consistent with the acquisition time, and shall be accompanied by the acquisition time in UTC, the Sun elevation and azimuth, the off-nadir angle and the coordinates of the footprint corners. The same orbit and attitude profiles shall be used in the coverage, power, thermal, imaging and astronomy analyses, so that the results are consistent with each other. Tools, models, assumptions and input data shall be documented, with the input files needed by the evaluators to reproduce the main results.

The proposal shall include a compliance statement that gives, for each provision containing “shall” in Sections 4 and 5, the status (compliant, partially compliant, non-compliant) and the reference to the supporting evidence. Any deviation shall be justified.

## 7.2 Deliverables

The applicant shall deliver the following items, in English:

- the optimised constellation architecture, with its revisit time and coverage;

- the satellites, the dates and the start and end times of the two reference passes;

- the power budget, with its tables and figures;

- the temperatures of faces, solar panels and battery;

- the separation between the satellites and their disposal delta-v;

- the simulated images of the daylight pass over the Colosseum;

- the light curve and the apparent magnitude from Matera;

- the compliance statement;

- the input and output data of the analyses, in open formats (CSV or JSON for numerical data; GeoTIFF or PNG with metadata for images).

The reports shall be delivered in PDF format: a final report, a technical note for each task and an executive summary of at most 10 pages.

# 8. Tasks and schedule

## 8.1 Tasks

The activity is organised in six tasks. In the first task the applicant performs the mission analysis and the optimisation of the constellation described in Section 4.3. The second task covers the design of the platform and of the payload, including the selection of components, the attitude control and the communications described in Sections 5.1, 5.2, 5.3 and 5.6. The third task is the power and thermal analysis of Sections 5.4 and 5.5. The fourth task is the simulation of the daylight imaging of Case A, and the fifth task is the analysis of the impact on astronomy and of Case B, as described in Section 5.7. In the sixth task the applicant consolidates the results, prepares the compliance statement and delivers the items of Section 7.2.

## 8.2 Milestones

The activity starts with a kick-off meeting at T0. A mid-term review takes place at T0 + 3 months to assess the constellation architecture and the platform selection. The final review takes place at T0 + 6 months on the basis of the final report and of the deliverables of Section 7.2.

# 9. Evaluation criteria

Proposals will be evaluated by a panel appointed by ASI, which may include external experts, on the following criteria, each scored from 0 to 5 and weighted as indicated.

Compliance with the call (30 percent). Extent to which the proposal meets the provisions of Sections 4 and 5 and provides the evidence of Section 7. A proposal scoring less than 3 on this criterion will not be considered further.

Technical quality of the design (25 percent). Soundness of the constellation optimisation, justification of the choice of components, consistency of the analyses with each other, and quality of the simulations.

Service to the cultural heritage monuments (25 percent). Revisit time and coverage achieved on each of the reference monuments of Section 4.2, assessed against the values and goals of Section 4.3. The panel will consider the performance on every monument and not only the average over the set, and will give credit to a balanced service between the northern targets (Milan, Venice) and the southern ones (Pompeii, Rome), and to the weakest monument being served close to the others. The panel will also consider the usefulness of the imagery for monument monitoring, based on the simulated acquisition over the Colosseum in Case A (footprint centred on the monument, illumination and viewing geometry suited to the observation of the structure), and the ability of the architecture to accommodate further monuments at other Italian latitudes without a redesign of the constellation.

Impact on astronomy (20 percent). Brightness of the satellites as seen from the ground, quality of the reflectance analysis and effectiveness of the mitigation measures.

In the case of equal scores, the proposal with the lowest impact on astronomy ranks first. Costs and lifecycle economics are outside the scope of this call and will not be evaluated.

# 10. Eligibility, submission and clarifications

Eligibility. Universities, research institutions and industries established in an ASI or ESA Member State may apply, alone or in a team. A team shall nominate a single prime applicant.

Format of the proposal. The proposal shall be submitted as a single PDF file written in English, with an executive summary of at most 2 pages and a technical part of at most 40 pages, annexes excluded. The supporting data of Section 7.2 shall be submitted as separate files.

Submission. The closing date and the submission portal are indicated in the announcement of the call. Proposals remain valid for 90 days after the closing date.

Clarifications. Questions shall be submitted in writing to the contact point indicated in the announcement, no later than three weeks before the closing date. Questions and answers will be made available to all applicants.